Waste Motor Oil / Vegetable Oil Refinery

This SDD Includes Interface and Electrical Design documentation.

Designed and built by: Corbin Probasco

August 2026

Temp sensors (10K NTC3950)

10K NTC3950 thermistor, resistance table computed from the Beta equation (R = R25 x exp(B x (1/T - 1/T25)), R25=10000 ohm, B=3950). ADC raw column assumes the divider 3.3V -> thermistor -> ADC pin -> 10k fixed resistor -> GND, 12-bit ADC (0-4095). Note the 25C row lands at exactly half-scale (2048), confirming the 10k fixed resistor is well matched to this thermistor. ADC reading rises as temperature rises with this wiring orientation, matching the level sender's convention below (higher ADC = more of what's being measured).

| Temp (C) | Temp (F) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- | --- |
| -10 | 14 | 58380 | 600 |
| -5 | 23 | 44040 | 757 |
| 0 | 32 | 33530 | 940 |
| 5 | 41 | 25940 | 1139 |
| 10 | 50 | 20176 | 1357 |
| 15 | 59 | 15838 | 1585 |
| 20 | 68 | 12540 | 1817 |
| 25 | 77 | 10000 | 2047 |
| 30 | 86 | 8041 | 2270 |
| 35 | 95 | 6505 | 2481 |
| 40 | 104 | 5301 | 2676 |
| 45 | 113 | 4349 | 2854 |
| 50 | 122 | 3588 | 3014 |
| 55 | 131 | 2979 | 3155 |
| 60 | 140 | 2487 | 3279 |
| 65 | 149 | 2088 | 3388 |
| 70 | 158 | 1762 | 3482 |
| 75 | 167 | 1493 | 3563 |
| 80 | 176 | 1271 | 3633 |
| 85 | 185 | 1087 | 3694 |
| 90 | 194 | 934 | 3745 |
| 95 | 203 | 805 | 3790 |
| 100 | 212 | 698 | 3828 |

Tank Level Sensors

Marine reed/float level sender, American/SAE standard, measured 240 ohm empty (float down) to 33 ohm full (float up). Divider uses a 100 ohm fixed resistor: 3.3V -> sender -> ADC pin -> 100 ohm -> GND. With this wiring orientation ADC reading rises as the tank fills. Table assumes resistance varies linearly with float position across the sender's travel; this is the standard assumption for this type of sender but has not been verified against the physical unit, calibrate against known fill levels before trusting absolute values. 12-bit ADC (0-4095).

| Fill (%) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- |
| 0 | 240.0 | 1204 |
| 10 | 219.3 | 1282 |
| 20 | 198.6 | 1371 |
| 30 | 177.9 | 1474 |
| 40 | 157.2 | 1592 |
| 50 | 136.5 | 1732 |
| 60 | 115.8 | 1898 |
| 70 | 95.1 | 2099 |
| 80 | 74.4 | 2348 |
| 90 | 53.7 | 2664 |
| 100 | 33.0 | 3079 |

STM32 Pin Assignments

GPIO Outputs - Arduino Header

| Signal | Pin | STM32 |
| --- | --- | --- |
| Tank 1 Solenoid | D2 | PA10 |
| Tank 2 Solenoid | D3 | PB3 |
| Tank 1 Heater | D4 | PB5 |
| Tank 2 Heater | D5 | PB4 |
| Nozzle Heater | D6 | PB10 |
| Fire Ext Extend | D7 | PA8 |
| Fire Ext Retract | D8 | PA9 |
| Centerfuge Motor | D9 | PC7 |

GPIO Inputs - Morpho Header CN10

| Signal | Pin |
| --- | --- |
| Flame Sensor 1 | PB1 |
| Flame Sensor 2 | PB2 |
| Flame Sensor 3 | PC5 |

ADC Inputs - Arduino Header

| Signal | Pin | STM32 |
| --- | --- | --- |
| Tank 1 Temp | A0 | PA0 |
| Tank 2 Temp | A1 | PA1 |
| Nozzle Temp | A2 | PA4 |
| Tank 1 Level | A3 | PB0 |
| Tank 2 Level | A4 | PC1 |

Serial - USART2

| Signal | Pin | STM32 |
| --- | --- | --- |
| TX | (Arduino)D10 | PB6 |
| RX | (MorphoCN7) | PB7 |

Raspberry Pi Pin Assignments
Serial USART2

| Signal | Pin | STM32 Connection |
| --- | --- | --- |
| TX | TXD Pin 8 | PB7 |
| RX | RXD Pin 10 | PB6 |
| Ground | GND Pin 6 | STM32 GND |

Pinout Diagrams

STM32 Nucleo-L476RG

Raspberry Pi 2 Model B