Temp Sensors

10K NTC3950 thermistor, resistance table computed from the Beta equation (R = R25 x exp(B x (1/T - 1/T25)), R25=10000 ohm, B=3950). ADC raw column assumes the divider 3.3V -> 10k fixed resistor -> ADC pin -> thermistor -> GND, 12-bit ADC (0-4095). Note the 25C row lands at exactly half-scale (2048), confirming the 10k fixed resistor is well matched to this thermistor. ADC reading falls as temperature rises with this wiring orientation; if the thermistor and fixed resistor are swapped, the relationship inverts.

| Temp (C) | Temp (F) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- | --- |
| -10 | 14 | 58380 | 3495 |
| -5 | 23 | 44040 | 3338 |
| 0 | 32 | 33530 | 3155 |
| 5 | 41 | 25940 | 2956 |
| 10 | 50 | 20176 | 2738 |
| 15 | 59 | 15838 | 2510 |
| 20 | 68 | 12540 | 2278 |
| 25 | 77 | 10000 | 2048 |
| 30 | 86 | 8041 | 1825 |
| 35 | 95 | 6505 | 1614 |
| 40 | 104 | 5301 | 1419 |
| 45 | 113 | 4349 | 1241 |
| 50 | 122 | 3588 | 1081 |
| 55 | 131 | 2979 | 940 |
| 60 | 140 | 2487 | 816 |
| 65 | 149 | 2088 | 707 |
| 70 | 158 | 1762 | 613 |
| 75 | 167 | 1493 | 532 |
| 80 | 176 | 1271 | 462 |
| 85 | 185 | 1087 | 401 |
| 90 | 194 | 934 | 350 |
| 95 | 203 | 805 | 305 |
| 100 | 212 | 698 | 267 |

Tank Level Sensors

Marine reed/float level sender, American/SAE standard, measured 240 ohm empty (float down) to 33 ohm full (float up). Divider uses a 100 ohm fixed resistor: 3.3V -> sender -> ADC pin -> 100 ohm -> GND. With this wiring orientation ADC reading rises as the tank fills. Table assumes resistance varies linearly with float position across the sender's travel; this is the standard assumption for this type of sender but has not been verified against the physical unit, calibrate against known fill levels before trusting absolute values. 12-bit ADC (0-4095).

| Fill (%) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- |
| 0 | 240.0 | 1204 |
| 10 | 219.3 | 1282 |
| 20 | 198.6 | 1371 |
| 30 | 177.9 | 1474 |
| 40 | 157.2 | 1592 |
| 50 | 136.5 | 1732 |
| 60 | 115.8 | 1898 |
| 70 | 95.1 | 2099 |
| 80 | 74.4 | 2348 |
| 90 | 53.7 | 2664 |
| 100 | 33.0 | 3079 |