Waste Motor Oil / Vegetable Oil Refinery

This SDD Includes Interface and Electrical Design documentation.

Designed and built by: Corbin Probasco

August 2026

Temp sensors (10K NTC3950)

10K NTC3950 thermistor, resistance table computed from the Beta equation (R = R25 x exp(B x (1/T - 1/T25)), R25=10000 ohm, B=3950). ADC raw column assumes the divider 3.3V -> thermistor -> ADC pin -> 10k fixed resistor -> GND, 12-bit ADC (0-4095). Note the 25C row lands at exactly half-scale (2048), confirming the 10k fixed resistor is well matched to this thermistor. ADC reading rises as temperature rises with this wiring orientation, matching the level sender's convention below (higher ADC = more of what's being measured). Three identical dividers are used, one per probe (Tank 1 Temp on A0/PA0, Tank 2 Temp on A1/PA1, Nozzle Temp on A2/PA4). Each probe has one lead on the shared 3.3V rail and the other lead on its own 10k fixed resistor (R3, R4, R5); the ADC pin taps the node between the probe and its resistor, and the far ends of all three resistors join at the common ground. The fixed resistors are 10k +/-5% (the NTC3950 probes are 1%), so expect roughly 1-2 C of reading error at room temperature unless the actual resistor values are measured and used in firmware.

| Temp (C) | Temp (F) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- | --- |
| -10 | 14 | 58380 | 600 |
| -5 | 23 | 44040 | 757 |
| 0 | 32 | 33530 | 940 |
| 5 | 41 | 25940 | 1139 |
| 10 | 50 | 20176 | 1357 |
| 15 | 59 | 15838 | 1585 |
| 20 | 68 | 12540 | 1817 |
| 25 | 77 | 10000 | 2047 |
| 30 | 86 | 8041 | 2270 |
| 35 | 95 | 6505 | 2481 |
| 40 | 104 | 5301 | 2676 |
| 45 | 113 | 4349 | 2854 |
| 50 | 122 | 3588 | 3014 |
| 55 | 131 | 2979 | 3155 |
| 60 | 140 | 2487 | 3279 |
| 65 | 149 | 2088 | 3388 |
| 70 | 158 | 1762 | 3482 |
| 75 | 167 | 1493 | 3563 |
| 80 | 176 | 1271 | 3633 |
| 85 | 185 | 1087 | 3694 |
| 90 | 194 | 934 | 3745 |
| 95 | 203 | 805 | 3790 |
| 100 | 212 | 698 | 3828 |

Tank Level Sensors

Marine reed/float level sender, American/SAE standard, measured 240 ohm empty (float down) to 33 ohm full (float up). Divider uses a 100 ohm fixed resistor: 3.3V -> sender -> ADC pin -> 100 ohm -> GND. With this wiring orientation ADC reading rises as the tank fills. Table assumes resistance varies linearly with float position across the sender's travel; this is the standard assumption for this type of sender but has not been verified against the physical unit, calibrate against known fill levels before trusting absolute values. 12-bit ADC (0-4095).

| Fill (%) | Resistance (ohm) | ADC raw (12-bit) |
| --- | --- | --- |
| 0 | 240.0 | 1204 |
| 10 | 219.3 | 1282 |
| 20 | 198.6 | 1371 |
| 30 | 177.9 | 1474 |
| 40 | 157.2 | 1592 |
| 50 | 136.5 | 1732 |
| 60 | 115.8 | 1898 |
| 70 | 95.1 | 2099 |
| 80 | 74.4 | 2348 |
| 90 | 53.7 | 2664 |
| 100 | 33.0 | 3079 |

Output Drivers and Actuators

Heaters (SSR-40DA). The Tank 1 and Tank 2 block heaters (200W, 120V AC) and the nozzle heater are switched by HiLetgo SSR-40DA solid state relays: control input 3-32V DC, output 24-380V AC. Each SSR control input is driven from its heater GPIO (Tank 1 Heater D4/PB5, Tank 2 Heater D5/PB4, Nozzle Heater D6/PB10) with the minus side on the common ground. These replace the SSR-40DD relays in the original design. The DD has a DC output stage and cannot switch an AC load. The 3.3V GPIO high level sits near the bottom of the DA's 3-32V input range, so confirm each relay switches reliably from the pin during bring-up.

Tank valves (motorized ball valves). Each tank has an upper and a lower valve (four total, Tank 1/2 Upper/Lower in the wiring diagram), each a U.S. Solid 1/4" motorized ball valve: 304 stainless steel, full port, normally closed, 9-24V AC/DC, 2-wire auto return. The valves run from the 12V supply. Both valves on a tank are switched together by that tank's 3.3V relay module, on the Tank 1 Valve (D2/PA10) and Tank 2 Valve (D3/PB3) outputs. Powering a valve opens it; removing power lets it return closed on its own, so a power loss or the fire-shutdown power cut leaves both tanks closed. These replace the 12V solenoid valves in the original design, which could not handle the dirty oil: they stuck and overheated almost immediately. A motorized ball valve takes time to travel between open and closed, unlike a solenoid, so firmware must wait out the measured travel time before treating the valve as open or closed.

STM32 Pin Assignments

GPIO Outputs - Arduino Header

| Signal | Pin | STM32 |
| --- | --- | --- |
| Tank 1 Valve | D2 | PA10 |
| Tank 2 Valve | D3 | PB3 |
| Tank 1 Heater | D4 | PB5 |
| Tank 2 Heater | D5 | PB4 |
| Nozzle Heater | D6 | PB10 |
| Fire Ext Extend | D7 | PA8 |
| Fire Ext Retract | D8 | PA9 |
| Centerfuge Motor | D9 | PC7 |

GPIO Inputs - Morpho Header CN10

| Signal | Pin |
| --- | --- |
| Flame Sensor 1 | PB1 |
| Flame Sensor 2 | PB2 |
| Flame Sensor 3 | PC5 |

ADC Inputs - Arduino Header

| Signal | Pin | STM32 |
| --- | --- | --- |
| Tank 1 Temp | A0 | PA0 |
| Tank 2 Temp | A1 | PA1 |
| Nozzle Temp | A2 | PA4 |
| Tank 1 Level | A3 | PB0 |
| Tank 2 Level | A4 | PC1 |

Serial - USART1

| Signal | Pin | STM32 |
| --- | --- | --- |
| TX | (Arduino)D10 | PB6 |
| RX | (MorphoCN7) | PB7 |

Raspberry Pi Pin Assignments
Serial USART1

| Signal | Pin | STM32 Connection |
| --- | --- | --- |
| TX | TXD Pin 8 | PB7 |
| RX | RXD Pin 10 | PB6 |
| Ground | GND Pin 6 | STM32 GND |

Pinout Diagrams

STM32 Nucleo-L476RG

Raspberry Pi 2 Model B